## USERS' GUIDE TO THE AUDIO-VISUAL EQUIPMENT

### Using The Audio-Visual Equipment

1. Turn ON both Power Switches underneath the Equipment box. (Labeled PDU 1 & PDU 2)

2. IMPORTANT- strictly follow switching ON sequence as below - (Item 0 if required),

Items 1, 2, 3, 4, as numbered on the Units themselves.

- 0 = WI/label microphone, only if using - switch on/off holding down button for 2/3 seconds till light on/off)

- 1 - Mixer Unit - switch ON - green light.

- 2 - Microphone/Mic Receivers Unit - (2 x Hand Microphone, 1 to use, 1 spare). Switch ON number 2 at this lower level, just above the Mixer Unit. Do NOT touch any knobs at higher level.

- 3 - CD & AUX (Optional switch ON)

- 4 - AMP - switch ON - big blue light. NB Check volume levels first- both Channels, A & B should be approximately on 5.

### Using The Microphones.

1. Pick up switched OFF Hand Microphone from shelf, distinguishing between Microphone 1 or 2 (spare) or 3 (WI Lapel microphone is very sensitive).

2. Move away from A/V Equipment Cabinet to edge of stage – to prevent feedback - switch on microphone, try sound level, holding 6 inches below mouth and adjust if necessary.

3. The master switch on the right for the Mixer Unit can be adjusted, as well as the volume knobs for Microphone 1 or 2 (approx. at 11 o'clock).

Using The Projector

1. ONLY use the Projector Remote Control - found in the Equipment Cabinet on the shelf with the Microphones.

2. Switch ON (top left button), by aiming Remote Control at Projector itself on high-level bar above stage. Leave alone for a couple of minutes to warm up.

3. The pull-down screen OR the wall can be used for projection.

4. On the side of the A/V Equipment Cabinet are the video input ports (HDMI, VGA and COMPOSITE} for laptop plugs - appropriate leads to be found on top of the A/V Equipment Cabinet.

5. Turning OFF the Projector- Press Power Standby button until you see the power off message on the screen. Then press again to confirm power off. The projector will enter its cooling down process - please leave it to do this for 10 minutes before switching the main power sockets off as this helps the lamp to cool down properly.

Using The TV.

- If you want to use TV, use only the TV Remote Control.

- The TV receiver may already be on, but the HDMI video source selector may have to be automatically switched to the Amazon Fire TV box & the easiest way to get the projector to output the TV is to use the Power button on the remote so it goes off - give it a few seconds until the light on the receiver goes RED- then switch the TV receiver back on.

- TV Receiver - green light = ON.

- If there are any issues, then look at HDMI video source selector switch at the bottom right of the A/V Equipment Cabinet and press the button on it until the RED LED is lit next to the TV.

Using The CD Player.

1. Plug appropriate cable into Aux/CD port.

2. Press CD/USB/Aux button. This toggles through each of the 3 options as source – check on the display screen for which option you have chosen.

3. Adjust volume level, as required, on Mixer Unit.

Turning Off A/V Equipment

1. Do everything strictly in reverse order, namely 4, 3, 2, 1, 0.

2. Then the 2 Power switches (PDU1 and 2) under the A/V Equipment Cabinet.

Projector Screen

If you use the projector screen please ensure that the screen is released and wound up again at the end of the presentation.